FIFO VERIFICATION PLAN

- Introduction

This comprehensive verification plan details the architecture, checking strategies, and coverage goals for the synchronous FIFO, directly reflecting the implemented SystemVerilog testbench infrastructure.

- Dut Summary

The Device Under Test (DUT) is a synchronous FIFO with these specifications:

Table 1: FIFO Configuration

| Parameter | Specification |
| --- | --- |
| FIFO Type | Synchronous FIFO |
| Data Width | 8 bits |
| FIFO Depth | 32 entries |
| Address/Pointer Width | 6 bits |
| Clock | Single clock |
| Reset | Active HIGH |
| Write Interface | wr_en, wdata |
| Read Interface | rd_en |
| Status | full, empty |
| Read Data | rdata |

Table 2: Port Description Of FIFO

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | System clock |
| rst | Input | 1 | Active HIGH reset |
| wr_en | Input | 1 | Write enable |
| rd_en | Input | 1 | Read enable |
| wdata | Input | 8 | Write data bus |
| rdata | Output | 8 | Read data bus |
| full | Output | 1 | FIFO full flag |
| empty | Output | 1 | FIFO empty flag |

- Key Features to Verify

The testbench must confirm the following core functional requirements:

- Data Integrity: The FIFO must store and retrieve 8-bit data strictly in First-In, First-Out order.

- Flag Generation: The full flag must assert when all 32 memory slots are occupied, and the empty flag must assert when zero items reside in the FIFO.

- Pointer Wraparound: The read and write pointers must correctly wrap around after 32 consecutive operations without corrupting data or flag logic.

- Overflow Protection: The FIFO must reject write attempts when the full flag is high.

- Underflow Protection: The FIFO must reject read attempts when the empty flag is high.

- Reset Behavior: When rst is asserted, the internal memory array (32 entries of 8 bits) and the read/write pointers (wr_ptr, rd_ptr) must clear to zero.

- Verification Architecture

Figure 1: Verification Framework

Figure 1 shows the verification framework. The generator creates transactions and passes them to the driver through a mailbox. The driver applies them to the DUT pins through the interface. The monitor samples the pins and sends each observed cycle to the scoreboard and the coverage collector.

- Component description

| Component | File | Role |
| --- | --- | --- |
| FIFO_INTERFACE | FIFO_INTERFACE.sv | Bundles clk, rst, wr_en, rd_en, wdata, rdata, full, empty. Driver clocking block drives on posedge with 1 ns output skew. Monitor clocking block samples on negedge. |
| FIFO_TRANSACTION | FIFO_TRANSACTION.sv | One bus cycle: wr_en, rd_en, wdata (random) plus observed rdata, full, empty. Constraints: at least one of wr_en / rd_en, write bias 70 percent. Has display and do_copy. |
| FIFO_GENERATOR | FIFO_GENERATOR.sv | Creates stimulus by test mode (random, write, read, order, empty, full, overflow) and puts it in the gen2drv mailbox. Sets done when finished. |
| FIFO_DRIVER | FIFO_DRIVER.sv | Applies reset (2 cycles high), idles the bus, then drives one transaction per clock from the mailbox. |
| FIFO_MONITOR | FIFO_MONITOR.sv | After reset, samples inputs and outputs every negedge and sends a copy to the scoreboard and coverage mailboxes. |
| FIFO_SCOREBOARD | FIFO_SCOREBOARD.sv | Reference model (SystemVerilog queue, depth 32). Checks read data, empty flag and full flag. Prints PASS or FAIL summary. |
| FIFO_COVERAGE | FIFO_COVERAGE.sv | Functional covergroups cg_txn and cg_fifo_state, sampled per monitored cycle. |
| FIFO_ASSERTIONS | FIFO_ASSERTIONS.sv | SVA properties, bound into the DUT with bind. |
| FIFO_ENVIRONMENT | FIFO_ENVIRONMENT.sv | Builds components, runs reset, runs all components, drains, prints report. |
| FIFO_TOP | FIFO_TOP.sv | Clock generation, DUT instance, bind, plusargs (TESTNAME, NUMTXN), simulation timeout. |

- Testbench flow

- Top reads TESTNAME and NUMTXN plusargs and creates the environment.

- Environment builds components, then the driver applies reset: rst high for 2 clock cycles, then low.

- Generator, driver, monitor, scoreboard and coverage start together.

- When the generator is done and the driver has driven every transaction, the environment idles the bus and waits 10 cycles so the last read is captured by the monitor and scoreboard.

- Environment prints generator, driver and monitor counts, scoreboard summary and coverage percentages, then top calls $finish.

- A 1 ms simulation timeout in the top module ends a hung simulation.

- Checking Strategy

  - Scoreboard Checks

| Check | Description |
| --- | --- |
| Read data | When a read is accepted (rd_en high and empty low), the oldest value is popped from the reference queue. In the next sample rdata must equal it. |
| Empty flag | Every sample: DUT empty must equal (reference queue size == 0). |
| Full flag | Every sample: DUT full must equal (reference queue size == 32). |
| Write accept | A write is pushed into the reference queue only if full was low in that sample (overflow protection). |
| Read accept | A read pops the reference queue only if empty was low in that sample (underflow protection). |
| Simultaneous read and write | Both are applied using the pre-cycle flags, matching DUT behavior. |
| Final summary | Prints total checks, read passes, failures and items left, then OVERALL RESULT PASS or FAIL. |

  - Assertions

| Assertion | Property |
| --- | --- |
| assert_empty_after_reset | After rst falls, empty is 1 on the next clock |
| assert_not_full_after_reset | After rst falls, full is 0 on the next clock |
| assert_full_empty_not_both | full and empty are never high together (disabled during reset) |

Assertions are bound into the DUT, so they check internal-visible port behavior on every clock without any testbench stimulus dependency

- Test Plan

| Test | Stimulus | Objective |
| --- | --- | --- |
| random | NUMTXN random transactions. At least one of wr_en / rd_en per cycle, 70 percent write bias, random wdata, simultaneous read and write allowed. | General data integrity, flags, mixed traffic, simultaneous read / write |
| write | NUMTXN consecutive writes, wdata = index. | Fill FIFO, reach full, reject extra writes |
| read | 10 writes (0xA0 + i), then 10 reads. | Basic write then read, data order, return to empty |
| order | 20 writes, then 50 times (read, write), then 20 reads. | Strict FIFO order and pointer wraparound (pointers pass 64 operations) |
| empty | 32 reads on empty FIFO. | Underflow protection, empty flag stays high |
| full | 32 writes. | Full flag assertion at exactly 32 entries |
| overflow | 32 writes, then 5 more writes (0xFF). | Overflow protection, extra writes rejected, stored data unchanged |

- Functional Coverage Plan

| Coverpoint / Cross | Bins | Purpose |
| --- | --- | --- |
| cp_empty | not_empty, is_empty | Empty flag seen both values |
| cp_full | not_full, is_full | Full flag seen both values |
| cp_wdata | zero (00), low (01-3F), middle (40-7F), high (80-FE), all_one (FF) | Data pattern spread |
| cp_op | idle (00), write_only (10), read_only (01), simul (11) | All operation types |
| cross_op_full | cp_op x cp_full | Every operation while full and not full |
| cross_op_empty | cp_op x cp_empty | Every operation while empty and not empty |
| cp_full_trans | became_full (0 to 1), no_longer_full (1 to 0) | Full flag set and release |
| cp_empty_trans | became_empty (0 to 1), no_longer_empty (1 to 0) | Empty flag set and release |